# Sensor Compatibility and Integration Guide

## Overview

The CAESER Water Levels Monitoring System is designed with a modular architecture that supports multiple sensor manufacturers and data logger types. This guide explains current compatibility, integration procedures, and expansion capabilities.

## Current Sensor Support

### ✅ Fully Supported: Solinst Data Loggers

#### Solinst Levelogger Series

- Models: Levelogger 5, Levelogger Edge, Levelogger Gold

- File Formats:

  - .xle (XML Logger Export) - Primary format

  - .lev (Legacy format) - Auto-converted to XLE

- Features:

  - Complete metadata preservation

  - Automatic barometric compensation

  - Temperature and pressure readings

  - GPS coordinates and installation data

  - Calibration parameters and corrections

#### Solinst Barologger Series

- Models: Barologger 5, Barologger Edge, Barologger Gold

- Integration:

  - Atmospheric pressure compensation

  - Master barometric data creation

  - Multi-logger synthesis

  - Temperature correlation analysis

#### Data Processing Capabilities

Solinst XLE File → Metadata Extraction → Quality Control → Database Storage

↓

Barometric Compensation → Water Level Calculation → Analysis

## 🔄 Expanding Compatibility

### Campbell Scientific (Implementation Ready)

- Target Models: CR1000X, CR6, CR300 series

- File Formats: CSV, TOA5, binary formats

- Integration Points:

  - LoggerNet data retrieval

  - Custom CSV parsing templates

  - Real-time telemetry integration

  - Modular data conversion utilities

### In-Situ (Planned Integration)

- Target Models: Level TROLL, Aqua TROLL series

- File Formats: Win-Situ CSV exports, binary data

- Features:

  - Multi-parameter logging (level, temperature, conductivity)

  - Vented and non-vented pressure transducers

  - Wireless telemetry capabilities

### OTT HydroMet (Future Expansion)

- Target Models: OTT PLS, OTT CTD series

- File Formats: HydroOffice exports, CSV

- Capabilities:

  - High-precision water level measurement

  - Multi-parameter environmental monitoring

  - European standard compliance

## Modular Architecture Design

### Data Import Framework

class DataImporter:

"""Base class for sensor data importers"""

def __init__(self, manufacturer, model):

self.manufacturer = manufacturer

self.model = model

self.supported_formats = []

def import_data(self, file_path):

"""Override in manufacturer-specific implementations"""

pass

def validate_data(self, raw_data):

"""Common validation logic"""

pass

def extract_metadata(self, raw_data):

"""Extract sensor and deployment metadata"""

pass

# Manufacturer-specific implementations

class SolinstImporter(DataImporter):

"""Solinst XLE and LEV file importer"""

def import_data(self, file_path):

if file_path.endswith('.xle'):

return self.parse_xle(file_path)

elif file_path.endswith('.lev'):

return self.parse_lev(file_path)

class CampbellImporter(DataImporter):

"""Campbell Scientific data importer"""

def import_data(self, file_path):

if file_path.endswith('.csv'):

return self.parse_csv(file_path)

elif file_path.endswith('.dat'):

return self.parse_dat(file_path)

### Sensor Configuration System

{

"sensors": {

"solinst_levelogger": {

"manufacturer": "Solinst",

"type": "pressure_transducer",

"file_formats": [".xle", ".lev"],

"metadata_fields": [

"serial_number", "location", "installation_date",

"calibration_date", "pressure_range", "accuracy"

],

"processing_pipeline": [

"parse_xml", "extract_metadata", "validate_readings",

"apply_calibration", "quality_control"

]

},

"campbell_cr1000": {

"manufacturer": "Campbell Scientific",

"type": "datalogger",

"file_formats": [".csv", ".dat", ".toa5"],

"metadata_fields": [

"station_name", "logger_id", "program_name",

"sampling_interval", "measurement_units"

],

"processing_pipeline": [

"parse_csv", "map_columns", "convert_units",

"validate_timestamps", "quality_control"

]

}

}

}

## Integration Procedures

### Adding New Sensor Support

#### Step 1: Create Sensor Profile

# config/sensors/new_sensor.json

{

"manufacturer": "Manufacturer Name",

"model": "Sensor Model",

"type": "pressure_transducer",

"file_formats": [".csv", ".txt"],

"column_mapping": {

"timestamp": "Date/Time",

"pressure": "Pressure (psi)",

"temperature": "Temperature (°C)"

},

"unit_conversions": {

"pressure": {"from": "psi", "to": "feet_h2o", "factor": 2.31},

"temperature": {"from": "celsius", "to": "fahrenheit", "formula": "C*9/5+32"}

}

}

#### Step 2: Implement Data Parser

class NewSensorImporter(DataImporter):

def __init__(self):

super().__init__("Manufacturer", "Model")

self.config = load_sensor_config("new_sensor.json")

def parse_data_file(self, file_path):

"""Parse manufacturer-specific data format"""

# Implementation specific to data format

pass

def map_columns(self, raw_data):

"""Map manufacturer columns to standard format"""

mapping = self.config["column_mapping"]

standardized_data = {}

for standard_col, manufacturer_col in mapping.items():

standardized_data[standard_col] = raw_data[manufacturer_col]

return standardized_data

def apply_unit_conversions(self, data):

"""Convert units to standard format"""

conversions = self.config["unit_conversions"]

for field, conversion in conversions.items():

if field in data:

data[field] = self.convert_units(data[field], conversion)

return data

#### Step 3: Register in System

# Register new sensor in the import system

SENSOR_IMPORTERS = {

'solinst': SolinstImporter,

'campbell': CampbellImporter,

'new_sensor': NewSensorImporter,  # Add new sensor

}

def get_importer(file_path):

"""Auto-detect sensor type and return appropriate importer"""

file_extension = os.path.splitext(file_path)[1].lower()

if file_extension in ['.xle', '.lev']:

return SENSOR_IMPORTERS['solinst']()

elif file_extension == '.csv':

# Additional logic to distinguish CSV formats

return detect_csv_format(file_path)

return None

## Data Format Standards

### Internal Data Format

All sensor data is converted to a standardized internal format:

{

"metadata": {

"sensor_id": "SN123456",

"manufacturer": "Solinst",

"model": "Levelogger 5",

"installation_date": "2024-01-15T10:30:00Z",

"location": {

"latitude": 35.1174,

"longitude": -89.9711,

"elevation": 331.2

}

},

"readings": [

{

"timestamp": "2024-01-15T10:30:00Z",

"pressure": 14.73,  # psi

"temperature": 15.2,  # Celsius

"water_level": null,  # Calculated after processing

"quality_flag": "good"

}

],

"calibration": {

"pressure_offset": 0.0,

"temperature_offset": 0.0,

"calibration_date": "2024-01-01T00:00:00Z"

}

}

### Quality Control Standards

QUALITY_CONTROL_RULES = {

"pressure": {

"range": {"min": 0, "max": 100},  # psi

"rate_of_change": {"max": 5.0},   # psi/hour

"statistical": {"z_score_threshold": 3.0}

},

"temperature": {

"range": {"min": -20, "max": 50},  # Celsius

"rate_of_change": {"max": 10.0},   # °C/hour

"seasonal": {"expected_range": {"winter": [-5, 25], "summer": [10, 35]}}

},

"temporal": {

"timestamp_continuity": True,

"duplicate_detection": True,

"future_date_check": True

}

}

## Calibration and Accuracy

### Multi-Point Calibration

def apply_calibration(raw_readings, calibration_points):

"""

Apply multi-point calibration to sensor readings

Args:

raw_readings: List of raw sensor values

calibration_points: List of (raw_value, true_value) pairs

Returns:

calibrated_readings: Corrected sensor values

"""

# Create calibration curve using polynomial fitting

raw_vals = [point[0] for point in calibration_points]

true_vals = [point[1] for point in calibration_points]

# Fit polynomial (typically 2nd or 3rd order)

coefficients = np.polyfit(raw_vals, true_vals, deg=2)

# Apply calibration to all readings

calibrated_readings = []

for reading in raw_readings:

calibrated_value = np.polyval(coefficients, reading)

calibrated_readings.append(calibrated_value)

return calibrated_readings

### Drift Correction

def detect_and_correct_drift(readings, manual_measurements):

"""

Detect sensor drift and apply corrections

Args:

readings: Continuous sensor readings

manual_measurements: Periodic manual validation measurements

Returns:

corrected_readings: Drift-corrected sensor values

"""

# Calculate drift over time

drift_points = []

for manual in manual_measurements:

# Find nearest sensor reading

nearest_reading = find_nearest_reading(readings, manual['timestamp'])

drift = manual['value'] - nearest_reading['value']

drift_points.append((manual['timestamp'], drift))

# Interpolate drift correction over time

drift_correction = interpolate_drift(drift_points, readings)

# Apply corrections

corrected_readings = []

for i, reading in enumerate(readings):

corrected_value = reading['value'] + drift_correction[i]

corrected_readings.append({

**reading,

'value': corrected_value,

'drift_correction': drift_correction[i]

})

return corrected_readings

## Testing and Validation

### Sensor Integration Testing

def test_sensor_integration(sensor_importer, test_file):

"""

Comprehensive testing of new sensor integration

Args:

sensor_importer: Sensor-specific importer class

test_file: Path to test data file

Returns:

test_results: Dictionary with test outcomes

"""

results = {

'file_parsing': False,

'metadata_extraction': False,

'data_validation': False,

'quality_control': False,

'database_integration': False

}

try:

# Test file parsing

raw_data = sensor_importer.parse_data_file(test_file)

results['file_parsing'] = True

# Test metadata extraction

metadata = sensor_importer.extract_metadata(raw_data)

results['metadata_extraction'] = len(metadata) > 0

# Test data validation

validated_data = sensor_importer.validate_data(raw_data)

results['data_validation'] = validated_data is not None

# Test quality control

qc_results = apply_quality_control(validated_data)

results['quality_control'] = True

# Test database integration

db_success = store_in_database(validated_data, metadata)

results['database_integration'] = db_success

except Exception as e:

results['errors'] = str(e)

return results

## Best Practices for Integration

### File Format Handling

- Robust Parsing: Handle variations in manufacturer file formats

- Error Recovery: Gracefully handle corrupted or incomplete files

- Format Validation: Verify file integrity before processing

- Encoding Support: Handle different text encodings (UTF-8, ASCII, etc.)

### Metadata Preservation

- Complete Records: Preserve all available sensor and deployment metadata

- Standardization: Map to consistent internal metadata schema

- Provenance: Track data source and processing history

- Validation: Verify metadata completeness and accuracy

### Performance Optimization

- Streaming Processing: Handle large files without excessive memory usage

- Batch Operations: Process multiple files efficiently

- Caching: Cache frequently accessed calibration and configuration data

- Parallel Processing: Utilize multiple CPU cores for large datasets

### Quality Assurance

- Automated Testing: Comprehensive test suites for each sensor type

- Validation Data: Maintain test datasets for regression testing

- Documentation: Clear documentation for integration procedures

- Version Control: Track changes to sensor configurations and parsers

## Future Expansion Roadmap

### Phase 1: Campbell Scientific Integration

- Timeline: Q2 2024

- Scope: CR1000X, CR6, CR300 series support

- Features: Real-time telemetry, LoggerNet integration

### Phase 2: In-Situ Support

- Timeline: Q3 2024

- Scope: Level TROLL, Aqua TROLL series

- Features: Multi-parameter logging, wireless connectivity

### Phase 3: Universal CSV Support

- Timeline: Q4 2024

- Scope: Generic CSV parser with user-configurable mapping

- Features: Custom column mapping, unit conversion, quality control

### Phase 4: Real-time Integration

- Timeline: Q1 2025

- Scope: Live data feeds from telemetry systems

- Features: API integration, real-time alerts, continuous monitoring

Next Steps: Continue to Integration Guide for detailed implementation procedures and examples.