# Recharge Calculation Methods

## Overview

The CAESER Water Levels Monitoring System implements multiple scientifically validated methods for groundwater recharge estimation. This comprehensive guide explains the algorithms, parameters, and implementation details for each method.

## 🌊 RISE Method (Water Table Fluctuation)

### Scientific Basis

The RISE (Recharge from Instantaneous water level riSE) method is based on the principle that increases in groundwater levels are directly related to recharge events. This method assumes that rapid water level rises following precipitation are primarily due to groundwater recharge.

### Theoretical Foundation

Recharge = Δh × Sy

Where:

- Δh = Water level rise (L)

- Sy = Specific yield (dimensionless)

### Algorithm Implementation

#### Event Detection

def detect_recharge_events(water_levels, timestamps, min_rise=0.1, min_duration=6):

"""

Detect recharge events based on water level rises

Args:

water_levels: Time series of water level measurements (feet)

timestamps: Corresponding timestamps

min_rise: Minimum rise to consider as event (feet)

min_duration: Minimum event duration (hours)

Returns:

events: List of detected recharge events

"""

events = []

rising_phase = False

event_start = None

baseline_level = None

for i in range(1, len(water_levels)):

current_level = water_levels[i]

previous_level = water_levels[i-1]

current_time = timestamps[i]

# Detect start of rising phase

if not rising_phase and current_level > previous_level:

rising_phase = True

event_start = current_time

baseline_level = previous_level

# Detect end of rising phase

elif rising_phase and current_level <= previous_level:

# Check if event meets criteria

total_rise = current_level - baseline_level

event_duration = (current_time - event_start).total_seconds() / 3600

if total_rise >= min_rise and event_duration >= min_duration:

events.append({

'start_time': event_start,

'peak_time': current_time,

'baseline_level': baseline_level,

'peak_level': current_level,

'total_rise': total_rise,

'duration_hours': event_duration

})

rising_phase = False

return events

#### Recharge Calculation

def calculate_rise_recharge(events, specific_yield=0.15, efficiency=1.0):

"""

Calculate recharge for each detected event

Args:

events: List of recharge events from detect_recharge_events()

specific_yield: Aquifer specific yield (dimensionless)

efficiency: Recharge efficiency factor (0-1)

Returns:

recharge_results: Detailed recharge calculations

"""

recharge_results = []

total_annual_recharge = 0

for event in events:

# Calculate recharge in inches

rise_feet = event['total_rise']

recharge_inches = rise_feet * 12 * specific_yield * efficiency

# Calculate recharge rate

duration_hours = event['duration_hours']

recharge_rate = recharge_inches / duration_hours  # inches/hour

event_result = {

'event_id': f"RISE_{event['start_time'].strftime('%Y%m%d_%H%M')}",

'start_time': event['start_time'],

'peak_time': event['peak_time'],

'rise_feet': rise_feet,

'recharge_inches': recharge_inches,

'recharge_rate_in_per_hr': recharge_rate,

'duration_hours': duration_hours,

'specific_yield_used': specific_yield,

'efficiency_used': efficiency

}

recharge_results.append(event_result)

total_annual_recharge += recharge_inches

return {

'events': recharge_results,

'total_annual_recharge_inches': total_annual_recharge,

'average_event_recharge': total_annual_recharge / len(events) if events else 0,

'number_of_events': len(events)

}

### Parameter Configuration

RISE_PARAMETERS = {

'specific_yield': {

'default': 0.15,

'range': [0.05, 0.35],

'description': 'Aquifer specific yield (fraction of water released per unit volume)',

'aquifer_types': {

'sand': 0.20,

'sandy_clay': 0.12,

'clay': 0.05,

'gravel': 0.25,

'limestone': 0.10

}

},

'min_rise_threshold': {

'default': 0.1,  # feet

'range': [0.02, 0.5],

'description': 'Minimum water level rise to consider as recharge event'

},

'min_duration': {

'default': 6,  # hours

'range': [1, 24],

'description': 'Minimum duration for valid recharge event'

},

'recharge_efficiency': {

'default': 1.0,

'range': [0.1, 1.0],

'description': 'Fraction of precipitation that becomes recharge'

}

}

## 📉 MRC Method (Master Recession Curve)

### Scientific Basis

The Master Recession Curve (MRC) method analyzes groundwater recession patterns to estimate recharge. This method assumes that deviations from the master recession curve indicate periods of additional recharge.

### Theoretical Foundation

The master recession curve represents the natural decline of groundwater levels in the absence of recharge:

dh/dt = -α × h^n

Where:

- h = Water level above some datum

- α = Recession coefficient

- n = Recession exponent (typically 1.0-2.0)

### Algorithm Implementation

#### Recession Period Identification

def identify_recession_periods(water_levels, timestamps, min_duration=72):

"""

Identify periods of continuous water level decline

Args:

water_levels: Time series of water level measurements

timestamps: Corresponding timestamps

min_duration: Minimum recession duration (hours)

Returns:

recession_periods: List of recession period data

"""

recession_periods = []

current_recession = None

for i in range(1, len(water_levels)):

current_level = water_levels[i]

previous_level = water_levels[i-1]

current_time = timestamps[i]

# Check if water level is declining

if current_level < previous_level:

if current_recession is None:

# Start new recession period

current_recession = {

'start_time': timestamps[i-1],

'start_level': previous_level,

'levels': [previous_level, current_level],

'times': [timestamps[i-1], current_time]

}

else:

# Continue current recession

current_recession['levels'].append(current_level)

current_recession['times'].append(current_time)

else:

# End current recession if it meets duration criteria

if current_recession is not None:

duration = (current_recession['times'][-1] - current_recession['start_time']).total_seconds() / 3600

if duration >= min_duration:

current_recession['end_time'] = current_recession['times'][-1]

current_recession['end_level'] = current_recession['levels'][-1]

current_recession['duration_hours'] = duration

current_recession['total_decline'] = current_recession['start_level'] - current_recession['end_level']

recession_periods.append(current_recession)

current_recession = None

return recession_periods

#### Master Recession Curve Creation

def create_master_recession_curve(recession_periods):

"""

Create master recession curve from multiple recession periods

Args:

recession_periods: List of recession periods

Returns:

master_curve: Master recession curve parameters and function

"""

# Normalize all recession periods to start at h=1.0 and t=0

normalized_data = []

for period in recession_periods:

levels = np.array(period['levels'])

times = np.array([(t - period['start_time']).total_seconds() / 3600

for t in period['times']])

# Normalize levels (h/h0)

normalized_levels = levels / levels[0]

# Store normalized data

for i, (t, h) in enumerate(zip(times, normalized_levels)):

if h > 0:  # Avoid log(0)

normalized_data.append({'time': t, 'level': h})

# Fit exponential decay curve: h(t) = exp(-α*t^n)

def recession_function(t, alpha, n):

return np.exp(-alpha * (t ** n))

# Extract data for curve fitting

times = [d['time'] for d in normalized_data]

levels = [d['level'] for d in normalized_data]

# Fit parameters using least squares

try:

popt, pcov = curve_fit(recession_function, times, levels,

bounds=([0, 0.5], [1, 3.0]),

p0=[0.01, 1.0])

alpha, n = popt

# Calculate goodness of fit

predicted = recession_function(np.array(times), alpha, n)

r_squared = 1 - np.sum((np.array(levels) - predicted)**2) / np.sum((np.array(levels) - np.mean(levels))**2)

return {

'alpha': alpha,

'n': n,

'r_squared': r_squared,

'function': lambda t: recession_function(t, alpha, n),

'normalized_data': normalized_data

}

except Exception as e:

return {'error': f"Curve fitting failed: {str(e)}"}

#### Recharge Estimation from Deviations

def calculate_mrc_recharge(water_levels, timestamps, master_curve):

"""

Calculate recharge based on deviations from master recession curve

Args:

water_levels: Complete water level time series

timestamps: Corresponding timestamps

master_curve: Master recession curve from create_master_recession_curve()

Returns:

recharge_results: MRC-based recharge estimates

"""

recharge_events = []

# Find periods where actual levels exceed predicted recession

for i in range(1, len(water_levels)):

current_level = water_levels[i]

current_time = timestamps[i]

# Look back to find last recession peak

recession_start_idx = find_last_recession_peak(water_levels, i)

if recession_start_idx is not None:

# Calculate time since recession started

time_since_peak = (current_time - timestamps[recession_start_idx]).total_seconds() / 3600

# Predict level using master curve

initial_level = water_levels[recession_start_idx]

predicted_ratio = master_curve['function'](time_since_peak)

predicted_level = initial_level * predicted_ratio

# Calculate deviation

deviation = current_level - predicted_level

# If deviation is positive and significant, consider it recharge

if deviation > 0.05:  # 0.05 feet threshold

# Convert deviation to recharge

specific_yield = 0.15  # Default value, should be configurable

recharge_inches = deviation * 12 * specific_yield

recharge_events.append({

'time': current_time,

'predicted_level': predicted_level,

'actual_level': current_level,

'deviation_feet': deviation,

'recharge_inches': recharge_inches,

'time_since_peak_hours': time_since_peak

})

# Aggregate annual recharge

total_recharge = sum(event['recharge_inches'] for event in recharge_events)

return {

'events': recharge_events,

'total_annual_recharge_inches': total_recharge,

'master_curve_parameters': {

'alpha': master_curve['alpha'],

'n': master_curve['n'],

'r_squared': master_curve['r_squared']

},

'number_of_events': len(recharge_events)

}

## ⚡ ERC Method (Enhanced Recharge Calculation)

### Scientific Basis

The Enhanced Recharge Calculation (ERC) method combines elements of both RISE and MRC methods while incorporating additional factors such as precipitation timing, soil moisture, and seasonal variations.

### Algorithm Implementation

#### Multi-Factor Recharge Analysis

def calculate_erc_recharge(water_levels, timestamps, precipitation_data=None,

soil_data=None, parameters=None):

"""

Enhanced recharge calculation using multiple factors

Args:

water_levels: Water level time series

timestamps: Corresponding timestamps

precipitation_data: Optional precipitation data

soil_data: Optional soil moisture data

parameters: ERC-specific parameters

Returns:

erc_results: Enhanced recharge calculations

"""

if parameters is None:

parameters = ERC_DEFAULT_PARAMETERS

# Step 1: Basic RISE calculation

rise_events = detect_recharge_events(water_levels, timestamps)

# Step 2: Recession analysis for baseline

recession_periods = identify_recession_periods(water_levels, timestamps)

master_curve = create_master_recession_curve(recession_periods)

# Step 3: Enhanced analysis with additional factors

enhanced_events = []

for event in rise_events:

enhanced_event = dict(event)

# Factor 1: Precipitation correlation

if precipitation_data:

precip_correlation = correlate_with_precipitation(

event, precipitation_data, parameters['precip_window_hours']

)

enhanced_event['precipitation_correlation'] = precip_correlation

enhanced_event['precipitation_efficiency'] = calculate_precip_efficiency(

event['recharge_inches'], precip_correlation['total_precip']

)

# Factor 2: Seasonal adjustment

seasonal_factor = calculate_seasonal_factor(

event['start_time'], parameters['seasonal_factors']

)

enhanced_event['seasonal_factor'] = seasonal_factor

# Factor 3: Antecedent conditions

antecedent_conditions = analyze_antecedent_conditions(

water_levels, timestamps, event['start_time'],

parameters['antecedent_period_days']

)

enhanced_event['antecedent_conditions'] = antecedent_conditions

# Factor 4: Soil moisture influence (if available)

if soil_data:

soil_influence = analyze_soil_moisture_influence(

event, soil_data, parameters['soil_influence_weight']

)

enhanced_event['soil_moisture_factor'] = soil_influence

# Calculate enhanced recharge estimate

base_recharge = event['recharge_inches']

enhancement_factor = (

seasonal_factor *

antecedent_conditions['adjustment_factor'] *

(soil_influence if soil_data else 1.0)

)

enhanced_event['enhanced_recharge_inches'] = base_recharge * enhancement_factor

enhanced_event['enhancement_factor'] = enhancement_factor

enhanced_events.append(enhanced_event)

# Calculate uncertainty estimates

uncertainty_analysis = calculate_uncertainty(enhanced_events, parameters)

return {

'events': enhanced_events,

'total_annual_recharge_inches': sum(e['enhanced_recharge_inches'] for e in enhanced_events),

'uncertainty_analysis': uncertainty_analysis,

'method_comparison': {

'rise_total': sum(e['recharge_inches'] for e in enhanced_events),

'erc_total': sum(e['enhanced_recharge_inches'] for e in enhanced_events),

'enhancement_ratio': sum(e['enhancement_factor'] for e in enhanced_events) / len(enhanced_events)

}

}

#### Precipitation Correlation Analysis

def correlate_with_precipitation(recharge_event, precipitation_data, window_hours=48):

"""

Correlate recharge events with precipitation data

Args:

recharge_event: Single recharge event

precipitation_data: Precipitation time series

window_hours: Time window to consider before event (hours)

Returns:

correlation_results: Precipitation correlation analysis

"""

event_start = recharge_event['start_time']

window_start = event_start - timedelta(hours=window_hours)

# Find precipitation within time window

relevant_precip = []

for precip_record in precipitation_data:

if window_start <= precip_record['timestamp'] <= event_start:

relevant_precip.append(precip_record)

if not relevant_precip:

return {'total_precip': 0, 'max_intensity': 0, 'correlation_strength': 0}

# Calculate precipitation statistics

total_precip = sum(p['amount'] for p in relevant_precip)

max_intensity = max(p['intensity'] for p in relevant_precip)

# Calculate lag time (time between precipitation peak and water level response)

precip_peak_time = max(relevant_precip, key=lambda x: x['intensity'])['timestamp']

lag_hours = (event_start - precip_peak_time).total_seconds() / 3600

# Estimate correlation strength based on lag time and amounts

correlation_strength = calculate_correlation_strength(

total_precip, max_intensity, lag_hours, recharge_event['total_rise']

)

return {

'total_precip': total_precip,

'max_intensity': max_intensity,

'lag_hours': lag_hours,

'correlation_strength': correlation_strength,

'precipitation_events': len(relevant_precip)

}

### Parameter Configuration

ERC_DEFAULT_PARAMETERS = {

'specific_yield': 0.15,

'precip_window_hours': 48,

'antecedent_period_days': 30,

'seasonal_factors': {

'spring': 1.2,  # Higher recharge efficiency

'summer': 0.8,  # Lower due to ET

'fall': 1.1,    # Moderate efficiency

'winter': 1.0   # Baseline

},

'soil_influence_weight': 0.3,

'uncertainty_factors': {

'measurement_error': 0.05,

'parameter_uncertainty': 0.15,

'method_uncertainty': 0.10

}

}

## 📊 Method Comparison and Validation

### Cross-Method Validation

def compare_recharge_methods(water_levels, timestamps, validation_data=None):

"""

Compare results from all three recharge calculation methods

Args:

water_levels: Water level time series

timestamps: Corresponding timestamps

validation_data: Optional validation data for comparison

Returns:

comparison_results: Comprehensive method comparison

"""

# Calculate recharge using all three methods

rise_results = calculate_rise_recharge(

detect_recharge_events(water_levels, timestamps)

)

recession_periods = identify_recession_periods(water_levels, timestamps)

master_curve = create_master_recession_curve(recession_periods)

mrc_results = calculate_mrc_recharge(water_levels, timestamps, master_curve)

erc_results = calculate_erc_recharge(water_levels, timestamps)

# Statistical comparison

comparison = {

'annual_recharge_inches': {

'RISE': rise_results['total_annual_recharge_inches'],

'MRC': mrc_results['total_annual_recharge_inches'],

'ERC': erc_results['total_annual_recharge_inches']

},

'number_of_events': {

'RISE': rise_results['number_of_events'],

'MRC': mrc_results['number_of_events'],

'ERC': erc_results['number_of_events']

},

'statistics': {

'mean': np.mean([

rise_results['total_annual_recharge_inches'],

mrc_results['total_annual_recharge_inches'],

erc_results['total_annual_recharge_inches']

]),

'std_dev': np.std([

rise_results['total_annual_recharge_inches'],

mrc_results['total_annual_recharge_inches'],

erc_results['total_annual_recharge_inches']

]),

'coefficient_of_variation': None  # Calculated below

}

}

# Calculate coefficient of variation

mean_recharge = comparison['statistics']['mean']

if mean_recharge > 0:

comparison['statistics']['coefficient_of_variation'] = (

comparison['statistics']['std_dev'] / mean_recharge

)

# Method reliability assessment

comparison['reliability_assessment'] = assess_method_reliability(

rise_results, mrc_results, erc_results, validation_data

)

return comparison

### Uncertainty Analysis

def calculate_uncertainty(recharge_results, parameters):

"""

Calculate uncertainty bounds for recharge estimates

Args:

recharge_results: Recharge calculation results

parameters: Method parameters including uncertainty factors

Returns:

uncertainty_bounds: Upper and lower confidence intervals

"""

base_recharge = sum(event['enhanced_recharge_inches'] for event in recharge_results['events'])

# Sources of uncertainty

measurement_error = parameters['uncertainty_factors']['measurement_error']

parameter_uncertainty = parameters['uncertainty_factors']['parameter_uncertainty']

method_uncertainty = parameters['uncertainty_factors']['method_uncertainty']

# Combined uncertainty (assuming independence)

total_uncertainty = np.sqrt(

measurement_error**2 +

parameter_uncertainty**2 +

method_uncertainty**2

)

# Calculate confidence intervals (95%)

confidence_interval = 1.96 * total_uncertainty * base_recharge

return {

'total_recharge': base_recharge,

'uncertainty_percentage': total_uncertainty * 100,

'confidence_interval_inches': confidence_interval,

'lower_bound': base_recharge - confidence_interval,

'upper_bound': base_recharge + confidence_interval,

'uncertainty_sources': {

'measurement': measurement_error * 100,

'parameters': parameter_uncertainty * 100,

'method': method_uncertainty * 100

}

}

## 🎯 Best Practices and Recommendations

### Method Selection Guidelines

- RISE Method:

  - Best for wells with clear, distinct recharge events

  - Requires good quality, high-frequency data

  - Most reliable in areas with episodic recharge

- MRC Method:

  - Suitable for wells with consistent recession patterns

  - Good for areas with more continuous recharge

  - Requires long-term data for reliable master curve

- ERC Method:

  - Most comprehensive but requires additional data

  - Best when precipitation and climate data available

  - Provides most robust uncertainty estimates

### Quality Control Recommendations

- Data Quality: Ensure high-quality, barometrically compensated data

- Manual Validation: Cross-check automated calculations with manual readings

- Parameter Sensitivity: Test sensitivity to key parameters

- Regional Calibration: Calibrate methods using local validation data

- Multiple Methods: Use multiple methods for cross-validation

### Reporting Standards

- Method Documentation: Clearly document which method(s) used

- Parameter Values: Report all parameter values and assumptions

- Uncertainty Bounds: Include uncertainty estimates in all reports

- Data Quality: Document data quality and limitations

- Validation: Include validation against independent measurements where available

Next Steps: Continue to File Formats to understand the data formats used throughout the system.