# Data Processing Workflows

## Overview

The CAESER Water Levels Monitoring System implements sophisticated data processing workflows that transform raw sensor readings into scientifically accurate water level measurements and analytical results.

## Data Processing Pipeline

### 1. Data Import Stage

#### XLE File Processing

Raw XLE File → XML Parser → Metadata Extraction → Data Validation → Database Storage

Steps:

- File Format Detection: Identify Solinst XLE format and version

- XML Parsing: Extract structured data from XML format

- Metadata Extraction:

  - Logger serial number and type

  - Installation location and dates

  - Calibration parameters

  - Measurement units and precision

- Data Validation:

  - Timestamp continuity checks

  - Value range validation

  - Quality flag assignment

- Database Storage: Insert validated data with proper indexing

#### Manual Reading Processing

Field Measurements → Validation → Tape Correction → Water Level Calculation → Database Storage

Processing Logic:

# Simplified water level calculation

def calculate_water_level(dtw_measurement, tape_correction, top_of_casing):

"""

Calculate water surface elevation from depth-to-water measurement

Args:

dtw_measurement: Depth to water (feet or meters)

tape_correction: Instrument-specific correction factor

top_of_casing: Elevation of measuring point

Returns:

water_level: Water surface elevation

"""

corrected_dtw = dtw_measurement + tape_correction

water_level = top_of_casing - corrected_dtw

return water_level

### 2. Quality Control Stage

#### Automated Quality Control Algorithms

##### Range Validation

-- Flag readings outside reasonable ranges

UPDATE water_level_readings

SET level_flag = 'error'

WHERE pressure < 0 OR pressure > 100  -- Example: 0-100 psi range

OR temperature < -20 OR temperature > 50;  -- -20°C to 50°C

##### Temporal Consistency Checks

def detect_anomalies(readings, threshold_std=3.0):

"""

Detect anomalous readings using statistical methods

Args:

readings: Time series of water level readings

threshold_std: Standard deviation threshold for anomaly detection

Returns:

anomaly_flags: Boolean array indicating anomalous readings

"""

# Calculate rolling statistics

rolling_mean = readings.rolling(window=24).mean()  # 24-hour window

rolling_std = readings.rolling(window=24).std()

# Flag readings beyond threshold

z_scores = (readings - rolling_mean) / rolling_std

anomaly_flags = abs(z_scores) > threshold_std

return anomaly_flags

##### Data Continuity Assessment

- Gap Detection: Identify missing data periods

- Sampling Rate Validation: Verify consistent measurement intervals

- Duplicate Detection: Remove or flag duplicate timestamps

### 3. Barometric Compensation

#### Master Barometric Data Creation

##### Multi-Logger Synthesis

def create_master_baro_data(barologger_readings):

"""

Create composite barometric pressure record from multiple loggers

Args:

barologger_readings: Dictionary of readings by serial number

Returns:

master_data: Unified barometric pressure time series

"""

# Align timestamps across all loggers

aligned_data = align_timestamps(barologger_readings)

# Quality-weighted averaging

weights = calculate_quality_weights(aligned_data)

master_pressure = weighted_average(aligned_data, weights)

# Gap filling using interpolation

master_data = fill_gaps(master_pressure, method='linear')

return master_data

##### Gap Filling Algorithms

- Linear Interpolation: For short gaps (< 6 hours)

- Seasonal Decomposition: For longer gaps using historical patterns

- Nearest Neighbor: When multiple barologgers are available

- Regional Data: Use nearby weather stations as backup

#### Water Pressure Calculation

def apply_barometric_compensation(transducer_pressure, baro_pressure):

"""

Remove atmospheric pressure effects from transducer readings

Args:

transducer_pressure: Raw pressure from transducer (absolute)

baro_pressure: Atmospheric pressure at same time

Returns:

water_pressure: Pressure due to water column only (gauge)

"""

# Convert to consistent units if necessary

transducer_psi = convert_to_psi(transducer_pressure)

baro_psi = convert_to_psi(baro_pressure)

# Calculate gauge pressure (water column only)

water_pressure = transducer_psi - baro_psi

return water_pressure

### 4. Water Level Calculation

#### Pressure to Water Level Conversion

def pressure_to_water_level(water_pressure, pressure_offset, elevation_offset):

"""

Convert water pressure to water surface elevation

Args:

water_pressure: Gauge pressure from water column (psi)

pressure_offset: Transducer calibration offset

elevation_offset: Elevation of transducer above datum

Returns:

water_level: Water surface elevation above datum

"""

# Convert pressure to feet of water (1 psi = 2.31 feet H2O)

pressure_head = (water_pressure - pressure_offset) * 2.31

# Calculate water surface elevation

water_level = elevation_offset + pressure_head

return water_level

#### Calibration Management

- Factory Calibration: Apply manufacturer calibration coefficients

- Field Calibration: Use manual measurements for field corrections

- Drift Correction: Account for sensor drift over time

- Multi-point Calibration: Use multiple reference points for accuracy

### 5. Data Integration and Merging

#### Multi-Source Data Consolidation

def merge_data_sources(transducer_data, manual_readings, telemetry_data):

"""

Combine data from multiple sources with proper precedence

Args:

transducer_data: Automated pressure transducer readings

manual_readings: Field measurements with water level meter

telemetry_data: Real-time telemetry system data

Returns:

consolidated_data: Merged dataset with quality indicators

"""

# Establish data source precedence

precedence = {

'manual': 1,      # Highest - direct field measurements

'transducer': 2,  # Medium - automated with corrections

'telemetry': 3    # Lowest - real-time but less accurate

}

# Merge with conflict resolution

merged_data = resolve_conflicts(

[transducer_data, manual_readings, telemetry_data],

precedence

)

return merged_data

#### Temporal Alignment

- Timestamp Standardization: Convert all data to UTC

- Interpolation: Align data to common time intervals

- Synchronization: Account for logger clock drift

### 6. Statistical Processing

#### Well Statistics Calculation

-- Generate comprehensive well statistics

INSERT INTO well_statistics (well_number, calculation_date, stats_json)

SELECT

well_number,

CURRENT_TIMESTAMP,

json_object(

'count', COUNT(*),

'mean_level', AVG(water_level),

'std_dev',

'min_level', MIN(water_level),

'max_level', MAX(water_level),

'range', MAX(water_level) - MIN(water_level),

'data_start', MIN(timestamp_utc),

'data_end', MAX(timestamp_utc)

) as stats_json

FROM water_level_readings

WHERE level_flag != 'error'

GROUP BY well_number;

#### Trend Analysis

def calculate_trends(water_levels, timestamps):

"""

Calculate water level trends using statistical methods

Args:

water_levels: Time series of water level measurements

timestamps: Corresponding timestamps

Returns:

trend_results: Dictionary with trend statistics

"""

# Convert timestamps to numeric for regression

time_numeric = convert_to_julian(timestamps)

# Linear regression for overall trend

slope, intercept, r_value, p_value = stats.linregress(time_numeric, water_levels)

# Seasonal decomposition

seasonal_component = seasonal_decompose(water_levels, period=365)

# Mann-Kendall trend test for non-parametric analysis

mk_trend, mk_p_value = mann_kendall_test(water_levels)

return {

'linear_trend': slope,

'r_squared': r_value**2,

'trend_significance': p_value,

'seasonal_amplitude': seasonal_component.seasonal.std(),

'mk_trend': mk_trend,

'mk_significance': mk_p_value

}

### 7. Recharge Calculations

#### RISE Method Processing

def calculate_rise_recharge(water_levels, timestamps, specific_yield=0.15):

"""

Calculate groundwater recharge using RISE method

Args:

water_levels: Time series of water level measurements

timestamps: Corresponding timestamps

specific_yield: Aquifer-specific yield parameter

Returns:

recharge_events: List of recharge events with quantities

"""

# Identify recharge events (water level rises)

events = detect_recharge_events(water_levels, timestamps)

# Calculate recharge for each event

recharge_events = []

for event in events:

# Water level rise in feet

rise_magnitude = event['peak_level'] - event['pre_event_level']

# Convert to recharge volume (rise × specific yield)

recharge_inches = rise_magnitude * 12 * specific_yield

recharge_events.append({

'start_date': event['start_date'],

'peak_date': event['peak_date'],

'rise_feet': rise_magnitude,

'recharge_inches': recharge_inches

})

return recharge_events

#### MRC Method Processing

def calculate_mrc_recharge(water_levels, timestamps):

"""

Calculate recharge using Master Recession Curve method

Args:

water_levels: Time series of water level measurements

timestamps: Corresponding timestamps

Returns:

mrc_results: Recession analysis and recharge estimates

"""

# Identify recession periods

recession_periods = identify_recessions(water_levels, timestamps)

# Create master recession curve

master_curve = create_master_recession_curve(recession_periods)

# Calculate recharge based on deviations from master curve

recharge_estimates = []

for period in recession_periods:

deviation = calculate_curve_deviation(period, master_curve)

recharge = deviation_to_recharge(deviation)

recharge_estimates.append(recharge)

return {

'master_curve': master_curve,

'recession_periods': recession_periods,

'recharge_estimates': recharge_estimates

}

### 8. Export and Visualization Processing

#### Data Preparation for Visualization

def prepare_visualization_data(raw_data, aggregation_level='daily'):

"""

Prepare data for efficient visualization and export

Args:

raw_data: High-resolution time series data

aggregation_level: Temporal aggregation ('hourly', 'daily', 'monthly')

Returns:

viz_data: Optimized data for plotting and export

"""

# Temporal aggregation for performance

if aggregation_level == 'daily':

agg_data = raw_data.resample('D').agg({

'water_level': ['mean', 'min', 'max'],

'temperature': 'mean'

})

# Quality flag summarization

quality_summary = summarize_quality_flags(raw_data)

# Statistical summaries

stats = calculate_summary_statistics(agg_data)

return {

'data': agg_data,

'quality': quality_summary,

'statistics': stats

}

#### Export Format Processing

- CSV Export: Tabular data with metadata headers

- JSON Export: Structured data for API consumption

- Database Export: SQLite files for sharing and archival

- GIS Export: Shapefiles and KML for mapping applications

## Error Handling and Recovery

### Data Integrity Checks

-- Comprehensive data integrity validation

SELECT

'Orphaned readings' as issue_type,

COUNT(*) as count

FROM water_level_readings wlr

LEFT JOIN wells w ON wlr.well_number = w.well_number

WHERE w.well_number IS NULL

UNION ALL

SELECT

'Invalid timestamps' as issue_type,

COUNT(*) as count

FROM water_level_readings

WHERE timestamp_utc > datetime('now', '+1 day')

OR timestamp_utc < datetime('1990-01-01');

### Recovery Procedures

- Automatic Correction: Fix common data issues automatically

- Manual Review: Flag issues requiring human intervention

- Rollback Capability: Undo problematic processing steps

- Backup Integration: Restore from backups when necessary

## Performance Optimization

### Batch Processing

def batch_process_files(file_list, batch_size=100):

"""

Process large numbers of files efficiently

Args:

file_list: List of files to process

batch_size: Number of files to process simultaneously

Returns:

results: Processing results and statistics

"""

results = []

for i in range(0, len(file_list), batch_size):

batch = file_list[i:i+batch_size]

# Parallel processing within batch

with ThreadPoolExecutor(max_workers=4) as executor:

batch_results = list(executor.map(process_single_file, batch))

results.extend(batch_results)

# Progress reporting

progress = (i + len(batch)) / len(file_list) * 100

update_progress(progress)

return results

### Memory Management

- Streaming Processing: Handle large datasets without loading entirely into memory

- Chunked Database Operations: Process data in manageable chunks

- Connection Pooling: Efficient database connection management

- Cache Management: Strategic caching of frequently accessed data

Next Steps: Continue to Calculations to understand the specific algorithms used for recharge calculations and other analytical methods.